# ESP32 Multi-Sensor Environmental Monitoring System

## 📋 Project Overview

This project implements an IoT-based environmental monitoring system using an ESP32-S2 microcontroller integrated with multiple sensors. Originally designed for ski slope monitoring, it continuously monitors environmental and motion parameters, providing real-time information through serial output and a web-based dashboard.

## 🎯 Project Objectives

- Interface ESP32 with BMP280 via I2C protocol

- Measure temperature, atmospheric pressure, and altitude

- Monitor snow moisture content for ski slope management

- Implement robust error handling with validation

- Display data on a web-based dashboard

- Extend system with MPU6050 motion sensor and HD-38 moisture sensor

- Enable automatic reconnection mechanisms

- Provide JSON API for external system integration

## 🔧 Hardware Components

### Microcontroller: ESP32-S2 WROVER Dev Module

- Dual-mode WiFi (Station + Access Point)

- 12-bit ADC for analog sensors

- Hardware I2C interface

- Operating voltage: 3.3V

### Sensors

#### 1. BMP280 – Environmental Sensor

- Temperature range: -40°C to +85°C

- Pressure range: 300–1100 hPa

- I2C address: 0x76 or 0x77 (auto-detected)

- Altitude calculated from barometric pressure

- Use case: Weather monitoring, snowmaking decisions

#### 2. MPU6050 – 6-Axis Motion Sensor

- 3-axis accelerometer (±8g range)

- 3-axis gyroscope (±500°/s range)

- Built-in temperature sensor

- I2C address: 0x68

- Use case: Slope movement detection, avalanche monitoring

#### 3. HD-38 – Capacitive Snow/Soil Moisture Sensor

- Analog output (0–8191 ADC range)

- Calibrated to 0–100% moisture

- Capacitive sensing technology

- Use case: Snow quality assessment for ski slopes

## 🔌 Wiring Configuration

### I2C Bus (Shared by BMP280 & MPU6050)

ESP32 GPIO 8  →  SDA (Serial Data)

ESP32 GPIO 9  →  SCL (Serial Clock)

### BMP280 Wiring

VCC  →  3.3V

GND  →  GND

SDA  →  GPIO 8

SCL  →  GPIO 9

CSB  →  3.3V (Enables I2C mode)

SDO  →  GND  (Sets address to 0x76)

### MPU6050 Wiring

VCC  →  3.3V

GND  →  GND

SDA  →  GPIO 8 (shared with BMP280)

SCL  →  GPIO 9 (shared with BMP280)

AD0  →  GND  (Sets address to 0x68)

### HD-38 Snow Moisture Sensor

VCC  →  3.3V

GND  →  GND

AO   →  GPIO 1 (Analog input)

## 💻 Software Architecture

### Programming Environment

- IDE: Arduino IDE 2.3.6

- Board: ESP32-S2 Dev Module

- Language: C++

- Baud Rate: 115200

### Required Libraries

Wire.h                    // I2C communication (built-in)

Adafruit_BMP280.h        // BMP280 driver

Adafruit_MPU6050.h       // MPU6050 driver

Adafruit_Sensor.h        // Unified sensor interface

WiFi.h                   // WiFi functionality (built-in)

WebServer.h              // HTTP server (built-in)

### Key Software Features

#### 1. Sensor Initialization & Auto-Detection

- Tries multiple I2C addresses automatically

- Configurable sampling rates and filters

- Graceful fallback if sensor not found

- Clear serial debug messages

#### 2. Data Acquisition

- Mode: Forced measurement (power-efficient)

- Interval: Every 2 seconds

- Method: Sequential multi-sensor polling

- Validation: Range checks on all readings

#### 3. Advanced Error Handling

- Data validation: Physical range checks

- NaN detection: Catches communication errors

- Auto-reconnection: Attempts reinit on failure

- Independent operation: One sensor failure doesn't affect others

- Debouncing: Prevents warning flicker (3-second interval)

#### 4. Communication Interfaces

Serial Monitor:

- Debugging output at 115200 baud

- Formatted sensor readings

- Status messages

- Error reporting

Web Dashboard:

- Clean responsive HTML

- Auto-refresh every 3 seconds

- Color-coded status indicators

- Mobile-friendly design

- Shows "N/A" instead of "nan" for invalid readings

JSON API:

- RESTful endpoint at /json

- Valid RFC 8259 compliant JSON

- Uses null for invalid values (not "nan")

- Structured data for integration

#### 5. Network Functionality

Dual WiFi Mode:

- Station Mode: Connects to existing network (FH_GAST)

- Access Point Mode: Creates backup network (ESP32_ENV, password: 12345678)

- Fallback: Automatic switch if primary fails

- Timeout: 10 seconds for connection attempt

## 📊 Data Processing

### BMP280 Measurements

- Temperature: Direct reading in °C (±1°C accuracy)

- Pressure: Converted from Pa to hPa (÷100)

- Altitude: Calculated using barometric formula

  - Formula: h = 44330 * (1 - (P/P₀)^(1/5.255))

  - P₀ = Sea level pressure (default: 1013.25 hPa)

  - Note: Adjust P₀ for local conditions for best accuracy

### MPU6050 Measurements

- Accelerometer: 3-axis linear acceleration (m/s²)

  - Stationary reading: ~10 m/s² (Earth's gravity)

  - Validation range: ±20 m/s²

- Gyroscope: 3-axis angular velocity (rad/s)

  - Stationary reading: ~0 rad/s

  - Validation range: ±100 rad/s (relaxed for movement)

- Temperature: Internal sensor reading (°C)

### Snow Moisture (HD-38)

- Raw ADC: 0–8191 (12-bit resolution)

- Calibration (for snow conditions):

  - Dry (no snow): 8191 → 0%

  - Wet (wet snow): 3000 → 100%

  - Note: Pure water reads ~1500, but snow is less conductive

Field Calibration Recommended:

- Measure raw value in exposed ground (no snow)

- Measure raw value in wettest available snow

- Update calibration constants in code

## 🌐 Web Dashboard Features

### HTML Interface

- Clean, professional layout

- UTF-8 charset for special characters (°C, m/s²)

- Responsive CSS design

- Auto-refresh via meta tag

- Calibration info displayed for transparency

### Status Indicators

- 🟢 Green: Normal operation

- 🟠 Orange: Sensor disconnected

- 🔴 Red: Warning threshold exceeded

### JSON API Structure

{

"bmp280": {

"connected": true,

"temperature": 23.73,

"pressure": 951.73,

"altitude": 525.28,

"sea_level_pressure": 1013.25

},

"mpu6050": {

"connected": true,

"accel": {"x": 10.18, "y": -0.39, "z": -1.84},

"gyro": {"x": -0.04, "y": 0.00, "z": 0.00},

"temperature": 24.15

},

"snow_moisture": {

"raw": 2232,

"percent": 89,

"calibration": {"dry": 8191, "wet": 3000}

},

"warning": "SNOW TOO WET / SLUSHY!"

}

## ⚠️ Warning System

### Alert Thresholds

| Parameter | Condition | Warning Message |
| --- | --- | --- |
| Temperature | > 45°C | HIGH TEMP! |
| Temperature | < -10°C | LOW TEMP! |
| Pressure | > 1050 hPa | HIGH PRESSURE! |
| Pressure | < 950 hPa | LOW PRESSURE! |
| Snow Moisture | < 20% | SNOW TOO DRY / NO SNOW! |
| Snow Moisture | > 80% | SNOW TOO WET / SLUSHY! |
| Sensor Status | Disconnected | [SENSOR] DISCONNECTED |

### Debouncing

- Warnings update every 3 seconds minimum

- Prevents flickering alerts

- Multiple warnings combined with "|" separator

## 🔬 Technical Implementation

### I2C Protocol

- Architecture: Master (ESP32) - Slave (sensors)

- Clock speed: 100 kHz (standard mode)

- Pull-ups: Internal resistors enabled

- Multi-drop: Supports multiple devices on same bus

- Addresses: 0x76/0x77 (BMP280), 0x68 (MPU6050)

### BMP280 Configuration

Mode:                 FORCED (power-saving)

Temperature sampling: X2 oversampling

Pressure sampling:    X16 oversampling (high accuracy)

IIR filter:          X16 (noise reduction)

Standby time:        500ms

### MPU6050 Configuration

Accelerometer range:  ±8g

Gyroscope range:     ±500°/s

Digital filter:      21 Hz bandwidth

Sampling:            Continuous

### Validation Logic

BMP280 Validation:

if (temperature < -40 || temperature > 85 ||

pressure < 300 || pressure > 1200 ||

isnan(temperature) || isnan(pressure)) {

// Mark sensor as disconnected

// Set values to NAN

// Attempt reconnection next cycle

}

MPU6050 Validation:

// Relaxed thresholds to prevent false disconnects

if (abs(accelX) > 20 || abs(accelY) > 20 || abs(accelZ) > 20 ||

abs(gyroX) > 100 || abs(gyroY) > 100 || abs(gyroZ) > 100 ||

isnan(accelX) || isnan(gyroX)) {

// Invalid reading - mark disconnected

}

Snow Moisture Validation:

if (soilRaw < 0 || soilRaw > 8191) {

// Out of ADC range - default to dry

}

// No disconnect detection (dry reading is valid for exposed ground)

## 🔄 Auto-Reconnection Mechanism

### Implementation

void readSensor() {

if (!sensorConnected) {

sensorConnected = initSensor();  // Attempt reconnection

if (!sensorConnected) {

setDataToNaN();

return;

}

}

// Normal reading operation

if (validationFails) {

sensorConnected = false;  // Will retry next cycle

}

}

### Benefits

- No manual reset: System recovers automatically

- Continuous operation: Works despite partial failures

- Self-healing: Reconnects within 2 seconds

- Independent sensors: One failure doesn't affect others

## 📈 Performance Characteristics

| Metric | Value | Notes |
| --- | --- | --- |
| Measurement frequency | 0.5 Hz (every 2s) | Balance of responsiveness & power |
| Temperature accuracy | ±1°C | BMP280 specification |
| Pressure accuracy | ±1 hPa | Absolute accuracy |
| Altitude resolution | ~1-8 meters | Depends on weather stability |
| WiFi range | 50–100m | Line-of-sight, varies by environment |
| Power consumption | 80–150mA | Active measurement mode |
| Boot time | ~3-5 seconds | Until first reading |
| Reconnection time | <2 seconds | After sensor reconnect |
| Web response time | <100ms | Dashboard load |

## 🎿 Ski Slope Application

### Use Cases

- Snowmaking Operations

  - Moisture < 20%: Trigger snowmaking

  - Temperature monitoring: Ensure cold enough for snow production

- Snow Quality Assessment

  - Moisture 20-80%: Optimal skiing conditions

  - Pressure: Weather prediction

- Safety Management

  - Moisture > 80%: Slushy/unsafe conditions

  - Motion sensor: Detect unusual slope movement

### Expected Snow Readings

| Snow Type | Raw ADC | Percentage | Status |
| --- | --- | --- | --- |
| No snow / bare ground | 7500-8191 | 0-10% | Needs snow |
| Dry powder | 6500-7500 | 10-25% | Good powder |
| Fresh packed | 5000-6500 | 25-45% | Excellent |
| Good skiing snow | 3500-5000 | 45-65% | Ideal |
| Wet/heavy snow | 2500-3500 | 65-80% | Marginal |
| Slushy/melting | 1500-2500 | 80-100% | Poor/unsafe |

## 🚀 Deployment Instructions

### Hardware Setup

- Connect all sensors according to wiring diagram

- Verify 3.3V and GND connections to all sensors

- Ensure CSB on BMP280 is connected to 3.3V (enables I2C mode)

- Verify SDO on BMP280 connected to GND (sets address 0x76)

- Verify AD0 on MPU6050 connected to GND (sets address 0x68)

### Software Setup

- Install Arduino IDE 2.3.6 or later

- Add ESP32 board support via Board Manager

- Install required libraries via Library Manager:

  - Adafruit BMP280 Library

  - Adafruit MPU6050

  - Adafruit Unified Sensor

- Select Board: ESP32S2 Dev Module

- Select correct COM port

- Upload sketch

- Open Serial Monitor at 115200 baud

### Initial Testing

- Verify all sensors detected in Serial Monitor

- Check displayed IP address

- Access web dashboard: http://[IP_ADDRESS]/

- Test JSON API: http://[IP_ADDRESS]/json

- Verify readings are realistic

### Field Calibration (for ski slope)

- Deploy sensor in target location

- Measure raw value with no snow (exposed ground)

- Measure raw value in wettest available snow

- Update calibration constants:

- const int SOIL_RAW_DRY = [your_dry_value];const int SOIL_RAW_WET = [your_wet_value];

- Re-upload code with updated values